| Potential hazard | Conse- quence | Likeli- hood | Risk | Control measure | Conse- quence | Likeli- hood | Final Risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tree collapse | 3 Spinal injury Broken bones Head injury | 3 | 9 | Make sure tree trunks are at least approx. 15cm diameter Make sure tree is healthy with no fungus, no considerable bark damage, has buds or foliage (Mar/April – Nov) Clear ground under hammock of stones, rocks, sticks and sharp objects Use natural V in tree wherever possible for attaching rope | 3 | 1 | 3 |
| Rope breakage | 3 Spinal injury Broken bones Head injury | 3 | 9 | Polypropylene rope 6mm diameter has breaking strain of approx. 500kg max load 66kg x 8 =533 kg Polypropylene rope diameter 10mm has breaking strain of 1200kg max load 160kg Make sure there is no slack in the hammock and end ropes as this will shock load the system | 3 | 1 | 3 |
| Wood support breakage | 3 Spinal injury Broken bones Head injury | 3 | 9 | Use wood support with diameter approx. 3- 4cm Ensure hammock is less than 20cm off ground when loaded Clear ground under hammock of stones, rocks, sticks and sharp objects Place shock absorbers (kneelers or cushions) under hammock in case of failure/fall |  |  |  |
| Cutting and sealing rope Using rope cutter gas lighters | 2 Minor burns | 2 | 4 | Remind participants not to touch hot elements, melted rope ends Fire glove available Water tub available for dousing hot rope end Supervised use of rope cutter Water, burns dressings available for minor burns For Adults all partial (blisters) and full thickness burns should go to A and E All burns in children go to hospital | 2 | 1 | 3 |
| Using Hammock | 3 Spinal injury Broken bones Head injury | 3 | 9 | Leader supervision at all times when in use Monitor use and calm group down if becoming too excited Enter and exit hammock carefully by standing astride the hammock Slowly lower self onto hammock Hammock will sink as rope stretches and knots tighten Leader could be first to try out hammock ( if it is safe for leader – it should be safe for a child) Consider using insulation tubes to protect tree from rope damage | 3 | 1 | 3 |
| Rope entanglement | 3 Strangulation Tourniquet effect | 3 | 9 | Leader supervision at all times when in use Make gaps between knots smaller than head size Leader to carry sharp scissors in order to cut rope immediately for entanglement If casualty’s limb has been tightly entangled for longer than 15mins, DO NOT untangle and call 999 | 3 | 1 | 3 |

Should activity take place, do the benefits outweigh the risks? Can the final risks be reduced to below 4? – Decision	Yes